- PURPOSE

  - The purpose of this document is to provide the specifications and requirements of the Cordless Tool Communication Interface for Deringer Electronic Switches and Zodiac Switch and Modules (SAM).

- DESCRIPTION

  - The Cordless Tool Communication Interface consists of:

    - Interface Box powered off universal AC (90V AC – 265V AC, 50 Hz – 60 Hz inputs) with a four-pin connector capable of mating with a cordless tool terminal and a standard RS232 interface for serial communication with a PC.  The Interface Box processes communication between the power tool and the PC to allow the transfer of data between the two devices.

    - Cordless Bootloader software that communicates to the power tool via the Interface Box and allows read/write access to the internal memories (Flash and EEPROM) of the power tool.

    - Personal computer to run the Cordless Bootloader software.

- FUNCTIONAL SPECIFICATIONS

  - Interface Box and Tool Connector

    - Schematic

    - The standard tool terminal has four inputs: B+, Thermistor (TH), Split Stack (SS) and B-.  The Interface Box connects to the tool via a four pin connector that supplies the signals to the four inputs of the tool terminal.  The inputs are used by the Interface Box as follows:

      - B+ and B- are used to supply power (15V) to the tool.

      - TH is used for data transmissions from the Interface Box to the tool (Interface Tx, Tool Rx).  Logic levels as seen on the terminal:

        - Logic level 0 is indicated by 0V.

        - Logic level 1 is indicated by 13V.

      - SS is used for data transmissions from the tool to the Interface Box (Interface Rx, Tool Tx).  Logic levels as seen on the terminal:

        - Logic level 0 is indicated by < 4.75V.

        - Logic level 1 is indicated by > 5.25V.

  - Communication Protocol

    - Communication from PC to Interface Box and from the Interface Box to the tool follows RS-232 communications protocol.

    - PC and Interface Box

      - 38,400 Baud

      - 10 bits (1 start bit, 8 data bits, and 1 stop bit)

      - No parity

      - No flow control

    - Interface Box and Tool

      - 9,467 Baud

      - 10 bits (1 start bit, 8 data bits, and 1 stop bit)

      - No parity

      - No flow control

  - Command Procedure

    - The process of executing a command generally follows the following procedure:

      - The PC initiates the command by sending an appropriate ASCII character to the Interface Box.  When the Interface Box receives an ASCII character that correlates with a command, it initiates the corresponding command with the connected tool.

      - As the command is being executed, the Interface Box serves as communication medium, transferring data from the PC to the Tool and vice versa.

      - While executing a command, if the Interface Box ever receives invalid data or times out while waiting for a device the command is aborted and a failure is indicated by sending an ASCII NAK character to the PC.

      - Upon successful completion of a command, the Tool will send an ASCII ETX character to the Interface Box which relays that back to the PC to signal the successful execution of the command.

    - Interface Box

      - When the Interface Box is not executing a command, it is constantly checking for the presence of an active tool by sending the Bootload command.  When a tool is connected to the Interface Box and powered up, it sees the Bootload command and goes into Bootloader instead of running the standard operational code.  Once the tool enters the Bootloader, it will respond appropriately to all commands coming to the tool from the Interface Box.

      - Interface Box State Machine

- COMMANDS

  - PC and Interface Box

    - “I” – IDENTIFY; used to identify the Interface Box.

    - “C” – STATUS; used to determine whether there is a tool connected to the Interface Box.

    - “V” – SAVE FLASH; used to load data into flash memory of Interface Box which will later be used to load into tools.

    - “R” – READ SAVED FLASH; used to read data that has been loaded into the flash memory of the Interface Box.

    - “V” – LOAD SAVED FLASH; used to load a page of flash memory into the tool from the data saved on the flash memory of the Interface Box.

  - Interface Box and Tool

    - “B” – BOOTLOAD; used to detect the presence of a tool connected to the Interface Box.

    - “D” – SET ADDRESS; used to set the flash address to read or write a page of flash on the tool.

    - “F” – READ EEPROM; used to read the EEPROM memory of the tool.

    - “E” – LOAD EEPROM; used to load specific values into the EEPROM of the tool.

    - “M” – LOAD Flash; used to load a page of flash memory into the tool.

    - “N” – READ Flash; used to read a page of flash memory from the tool.

  - Command Diagrams:

“B”

“B”

“C”

“C”

Interface Box

Tool

PC

S

ETX

“I”

“I”

X

ETX

Y

IDENTIFY

STATUS

CHECK SWITCH

S = 0 if no tool is connected

1 if tool is connected

X.Y = Firmware version of the interface box

X = Major value

Y = Minor value

TOOL COMS LED turns on if response is received from tool.

TO = 4ms

Timeout = 245ms

Timeout = 20ms

“D”

“D”

Interface Box

Tool

PC

ETX

“D”

“D”

AH

AH

AH

AL

AL

AL

SET ADDRESS

AH:AL = address

Timeout = 245ms

Timeout = 20ms

ETX

“E”

“E”

Dn

ETX

“E”

ETX

“E”

Dn

NH

NH

NL

NL

NL

NH

Dn

CH

CL

CH

CL

Interface Box

Tool

PC

LOAD EEPROM

NH:NL = number of bytes

D = data byte

CH:CL = checksum

Timeout = 245ms

Timeout = 20ms

Dn

D0

D0

D0

D0

…

…

“F”

“F”

NH

D1 … Dn

NL

ETX

“F”

NL

ETX

“F”

NH

D1 … Dn

Interface Box

Tool

PC

READ EEPROM

NH:NL = number of bytes

D = data byte

Timeout = 245ms

Timeout = 20ms

“V”

AH

“V”

AH

Interface Box

Tool

PC

SAVE FLASH

AL

AL

D1 … Dn

D1 … Dn

ETX

AH:AL = address

NH:NL = number of bytes

D = data byte

Timeout = 245ms

“R”

AH

“R”

AH

Interface Box

Tool

PC

READ SAVED FLASH

AL

AL

D1 … Dn

ETX

NH:NL = number of bytes

D = data byte

Timeout = 245ms

“W”

AH

“W”

AH

Interface Box

Tool

PC

LOAD SAVED FLASH

AL

AL

NH

NH

NL

NL

AH:AL = address

NH:NL = number of bytes

“M”

ETX

CH

CL

……

ETX

CH

CL

Load Flash command with NH:NL bytes of data coming from interface box flash  memory at address AH:AL.

Timeout = 245ms

Timeout = 20ms

“M”

“M”

D1 … Dn

ETX

“M”

ETX

“M”

D1 … Dn

NH

NH

NL

NL

NL

NH

D1 … Dn

CH

CL

CH

CL

Interface Box

Tool

PC

LOAD FLASH

NH:NL = number of bytes

D = data byte

CH:CL = checksum

Timeout = 245ms

Timeout = 20ms

“N”

“N”

NH

D1 … Dn

NL

ETX

“N”

NL

ETX

“N”

NH

D1 … Dn

Interface Box

Tool

PC

READ FLASH

NH:NL = number of bytes

D = data byte

Timeout = 245ms

Timeout = 20ms

“D”

“D”

Interface Box

Tool

PC

ETX

“D”

“D”

0xFF

0xFF

0xFF

0xFF

0xFF

0xFF

MODULE ID

Timeout = 245ms

Timeout = 20ms

ETX

“N”

NL

“N”

NH

ETX

D1 … Dn

“N”

“N”

NH

NL

D1 … Dn

ETX

NH:NL = number of bytes

“M”

“M”

0x00

ETX

“M”

ETX

“M”

0x00

0x00

0x00

0x01

0x01

0x01

0x00

0x00

0x00

0x00

0x00

Interface Box

Tool

PC

ERASE FLASH

NH:NL = number of bytes

D = data byte

CH:CL = checksum

Timeout = 245ms

Timeout = 20ms

0x00

  - Procedures

    - Loading Flash Memory

      - Loading flash memory directly is a two step process:

        - SET ADDRESS is used to set the address for the page of flash that will be loaded in the tool.

        - LOAD FLASH is used to send each byte that will be saved in the specified page of flash memory in the tool.

Or

ERASE FLASH is used to erase a page of flash memory starting at the set address.

    - Reading Flash Memory

      - Reading flash memory is a two step process:

        - SET ADDRESS is used to set the address of the page of flash that will be read from the tool.

        - READ FLASH is used to send each byte stored in the specified page of flash memory from the tool.

    - Loading Flash Memory from Interface Box

      - The follow procedures are used to load flash memory onto a tool using data that has been already saved in the Interface Box.  This is done to reduce the total time it takes to load flash memory onto the tool since it reduces the amount of communication necessary.

      - Saving Data onto Interface Box

        - SAVE FLASH is used to save one page of data (128 bytes) that will be used to load the flash memory of a tool.

        - READ SAVED FLASH is used to read one page of data (128 bytes) that has been saved into the flash memory of the Interface Box.

      - Loading Data from Interface Box

        - SET ADDRESS is used to set the address of the page of flash that will be loaded in the tool.

        - LOAD SAVED FLASH is used to load the flash memory of a tool using data that has been saved in the flash memory of the Interface Box.

    - Module ID

      - The following procedure is used to get the Module ID string and information if the module is setup to respond to the command.

        - SET ADDRESS with the address set to 0xFFFF.

        - READ FLASH will respond with Module ID response which starts with a 7 character ID string and then the remaining bytes contain other module information.

- PACKET COMMUNICATION

  - With packet communication the interface box remains unaware of the underlying protocol and acts only as a medium through which packets transfer as the PC and tool communicate.  The general protocol of packet communication is as follows:

    - PC sends packet to tool.

    - Tool responds by sending a packet in reply.

  - Commands

    - “U” – CHANGE BAUD; change the communication baud rate between Interface Box and tool while packet communication takes place.

    - “*” – PACKET COMMUNICATION; initiate packet communication between PC and tool.

  - Protocol

    - Packet communication is initiated by sending the “*” command.  With this command, the interface box transfers a packet from the PC to tool and then transfers the reply packet from tool to PC.  Once this is completed the Interface Box returns to the Check Checked state.

    - A packet as the following format:

COMM PACKET

| Byte offset |  |  |
| --- | --- | --- |
| 0 | CMD (1 byte) | Data Length, NH:NL (2 bytes) |
| 3 | Data (data length bytes) |  |

CMD = 1 byte ASCII character to represent a command.

Data Length = 2 byte value, length of data that will be sent in the rest of the packet.

Data = Data length number of bytes.

Minimum size = 3, 1 CMD byte + 2 Data length bytes (data length value = 0)

    - With packet communication, it is possible to change the communication baud rate between the interface box and tool from the default setting of 9,467 baud.  This is done through the CHANGE BAUD command.

  - Command Diagrams:

“U”

“U”

Interface Box

Tool

PC

ETX

D

D

CHANGE BAUD

D: baud delay value

9,457 Baud = 137

19,200 Baud = 66

28,800 Baud = 42

38,400 Baud = 31

57,600 Baud = 19

115,200 Baud = 8

Timeout = 245ms

Timeout = 20ms

“*”

“*”

ETX

PACKET

PACKET COMM

Timeout = 245ms

Timeout = 20ms

“*”

“*”

Change baud rate to higher speed if set by CHANGE BAUD

PACKET

PACKET

PACKET

Change baud rate back to default.

“M”

D1 … Dn

“M”

NH

NL

NL

NH

D1 … Dn

CH

CL

Interface Box

Tool

PC

LOAD FLASH

NH:NL = number of data bytes

D = data byte

CH:CL = checksum

Timeout = 245ms

Timeout = 295ms

“M”

0

2

CH

CL

“M”

0

2

“*”

“*”

“*”

“*”

Change baud rate to higher speed.

Change baud rate to higher speed.

Change baud rate back to 9600.

Change baud rate back to 9600.

“M”

“M”

0

0

0

0

CH

CL

Interface Box

Tool

PC

ERASE FLASH

NH:NL = number of data bytes

D = data byte

CH:CL = checksum

Timeout = 245ms

Timeout = 295ms

“M”

0

2

CH

CL

“M”

0

2

“*”

“*”

“*”

“*”

Change baud rate to higher speed.

Change baud rate to higher speed.

Change baud rate back to 9600.

Change baud rate back to 9600.

“N”

NH

D1 … Dn

NL

“N”

Interface Box

Tool

PC

READ FLASH

NH:NL = number of bytes

D = data byte

Timeout = 245ms

Timeout = 295ms

0

0

“N”

0

0

“N”

NH

D1 … Dn

NL

“*”

“*”

“*”

“*”

Change baud rate to higher speed.

Change baud rate to higher speed.

Change baud rate back to 9600.

Change baud rate back to 9600.

  - CHANGE HISTORY

| ECN # | REV # | DATE | BY | REASON |
| --- | --- | --- | --- | --- |
|  | A | 12/10/10 | Wing Lin | Creation of document. |
|  | B | 09/23/11 | Wing Lin | Updated interface box command diagrams to be clearer. |
|  | C | 10/04/11 | Wing Lin | Fixed diagrams for Set Address, Load EEPROM and Load Flash after James Goble found errors. |
|  | D | 07/09/12 | Wing Lin | Added module ID command. |
|  | E | 01/28/12 | Wing Lin | Added documentation for packet communication and thermistor/split stack interface box schematic. |
|  | F | 03/14/14 | Wing Lin | Correctly moved Load Saved Flash to PC and Interface Box (4.1). Updated Load EEPROM diagram to make it more obvious that each data byte sent needs to be echo’ed immediately once the tool receives it. Added Erase Flash packet command. |